Integrity Technologies, Inc.

1100 Hoods Mill Road

Commerce, Ga. 30529

(706) 335-3301 - Voice

(706) 335-3307 - Fax

info@integrity-technologies.net - Email

www.integrity-technologies.net - Web

Sample Testability Report

Board: D:\\SAMPLE.GWK

Time: 09:33:50

Date: 12-28-2001

Testability Scout Version: 1.1.2

Table of Contents

Fixturing Analysis	2

Single-sided bottom fixture	2

Probe Land Size Analysis	2

Inaccessible Nets Analysis	2

Probe Collision Analysis	2

Netlist	3

Netlist Summary	3

Bottom Side Inaccessible Nets	3

Bottom Side Accessible Nets	3

Potential Power/Ground Nets	4

Netlist Table	5

Parts	8

Part Summary and Pin Coverage	8

Parts with  40 or more pins	8

Parts with  16-39 pins	8

Parts with 4-15 pins	8

3-Pin Parts	9

2-Pin Parts	9

1-Pin Parts	11

# Fixturing Analysis

## Single-sided bottom fixture

| Total number of nets connected to parts | 133 |  |
| --- | --- | --- |
| Bottom side accessible nets | 47 | 35% |
| Bottom side inaccessible nets | 86 | 65% |
| Nets only accessible through SMD part lands, excludes no connects | 0 | 0% |

### Probe Land Size Analysis

| Nets where the largest accessible point is at least 35 mils | 47 | 35% |
| --- | --- | --- |
| Nets where the largest accessible point is at least 25 mils and less than 35 mils | 0 | 0% |
| Nets where the largest accessible point is less than 25 mils (ruled out as totally inaccessible) | 0 | 0% |

### Inaccessible Nets Analysis

| Nets with probe access covered by parts, excluding inactive part pins | 0 | 0% |
| --- | --- | --- |
| Nets consisting of single, inactive, SMD part pins (no connects) | 28 | 21% |
| Nets with probe access covered by solder mask | 0 | 0% |
| Top side only nets | 86 | 65% |

### Probe Collision Analysis

For nets on the Bottom side with an accessible point larger than 35 mils:

| Nets that may use 100 mil probes | 2 | 2% |
| --- | --- | --- |
| Nets that may need 70 mil probes | 45 | 34% |
| Nets that may need 50 mil probes | 0 | 0% |
| Nets that may need sub-50 mil probes | 0 | 0% |

# Netlist

## Netlist Summary

| Total number of nets in job | 145 |  |
| --- | --- | --- |
| Nets in job not connected to parts (fiducials, tooling holes, thieving pads, missing parts) | 12 |  |
| Total number of nets connected to parts | 133 |  |
| Total number of active nets (Nets connected between parts) | 105 | 79% |
| Total number of single, inactive(no connect), SMD part pin nets | 28 | 21% |

| Total number of accessible nets | 47 | 35% |
| --- | --- | --- |
| Total number of inaccessible nets | 85 | 64% |
| Bottom side accessible nets | 47 | 35% |
| Top side accessible nets | 0 | 0% |
| Top side only nets | 86 | 65% |
| Bottom side only nets | 0 | 0% |
| Potential Power/Ground Nets | 2 |  |

## Bottom Side Inaccessible Nets

86 nets are not present on the bottom side (Top side nets only):

| net | pin ref. | net | pin ref. | net | pin ref. | net | pin ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | C30.1 | 29 | C43.1 | 51 | R9.2 | 76 | U3.13 |
| 8 | R32.1 | 30 | C44.1 | 52 | R10.2 | 79 | U3.14 |
| 9 | C6.1 | 31 | C45.1 | 53 | R11.2 | 81 | U3.15 |
| 10 | C10.1 | 32 | CR5.1 | 54 | R13.1 | 82 | U3.17 |
| 11 | L3.1 | 33 | CR5.2 | 55 | R14.1 | 83 | U3.18 |
| 12 | C14.1 | 34 | CR5.3 | 56 | R16.1 | 84 | U3.19 |
| 13 | C15.1 | 35 | CR6.2 | 57 | R20.1 | 85 | U3.20 |
| 14 | C16.1 | 36 | CR6.3 | 58 | R21.1 | 86 | U3.21 |
| 15 | C16.2 | 37 | L1.1 | 59 | R22.1 | 87 | U3.22 |
| 16 | C20.2 | 38 | L2.1 | 60 | R27.2 | 88 | U3.26 |
| 17 | C21.1 | 39 | L3.2 | 61 | R29.1 | 89 | U11.1 |
| 18 | C22.1 | 40 | L4.1 | 62 | R30.2 | 90 | U11.2 |
| 19 | C22.2 | 41 | L8.2 | 63 | R37.2 | 91 | U11.9 |
| 20 | C23.1 | 42 | L9.2 | 64 | R42.1 | 92 | U15.1 |
| 21 | C25.1 | 43 | Q1.1 | 65 | R53.2 | 98 | U15.6 |
| 22 | C31.1 | 44 | Q4.1 | 66 | R54.1 | 99 | U15.7 |
| 23 | C32.1 | 45 | Q10.1 | 67 | U2.4 | 100 | U16.7 |
| 24 | C32.2 | 46 | Q14.3 | 68 | U2.5 | 101 | U17.7 |
| 25 | C34.1 | 47 | Q15.3 | 69 | U2.17 | 102 | U18.7 |
| 26 | C36.1 | 48 | Q17.1 | 70 | U2.19 | 112 | U19.7 |
| 27 | C37.1 | 49 | Q17.3 | 71 | U3.1 |  |  |
| 28 | C42.1 | 50 | R2.1 | 72 | U3.12 |  |  |

## Bottom Side Accessible Nets

| net | pin ref. | net | pin ref. | net | pin ref. | net | pin ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C1.2 | 103 | C11.1 | 117 | R12.2 | 136 | U18.1 |
| 3 | C7.1 | 104 | U2.18 | 118 | R8.2 | 137 | U19.1 |
| 73 | C7.2 | 105 | U2.11 | 119 | R7.2 | 139 | U17.1 |
| 74 | U17.5 | 106 | U2.12 | 123 | Q6.3 | 140 | C28.2 |
| 75 | L7.1 | 107 | U2.13 | 124 | L10.1 | 141 | Q16.3 |
| 77 | U18.5 | 108 | U2.14 | 128 | U2.6 | 143 | Q6.1 |
| 78 | U19.5 | 109 | U2.15 | 129 | C5.2 | 144 | Q3.3 |
| 80 | U16.5 | 110 | U2.16 | 130 | Q3.1 | 145 | Q4.3 |
| 93 | C18.1 | 111 | R6.2 | 131 | Q5.1 |  |  |
| 94 | U2.3 | 113 | C13.1 | 132 | R18.2 |  |  |
| 95 | C1.1 | 114 | C2.1 | 133 | Q2.2 |  |  |
| 96 | R52.1 | 115 | C17.1 | 134 | C34.2 |  |  |
| 97 | R55.1 | 116 | R5.2 | 135 | L7.2 |  |  |

## Potential Power/Ground Nets

| net | pin ref. | net | pin ref. | net | pin ref. | net | pin ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C1.2 | 3 | C7.1 |  |  |  |  |

## Netlist Table

| Net Number | Net Name | Bottom Side Accessible Points | Generic CAD Netlist Description |
| --- | --- | --- | --- |
| 1 | NET1 | 1 | C1.2 C2.2 C3.2 C4.2 C5.1 C6.2 C13.2 C14.2 C15.2 C17.2 C18.2 C19.2 C20.1 C21.2 C23.2 C24.2 C25.2 C26.2 C27.2 C28.1 C29.1 C30.2 C31.2 C35.1 C36.2 C37.2 C38.2 C39.2 C40.2 C41.2 C42.2 C43.2 C44.2 C45.2 CR4.2 CR4.3 Q3.2 Q4.2 Q5.2 Q6.2 Q7.2 Q8.2 Q9.2 Q12.3 Q14.2 Q15.2 Q17.2 Q18.2 R16.2 R19.2 R26.1 R29.2 R31.1 R34.1 R39.2 R51.2 R52.2 U2.7 U2.8 U5.4 U6.4 U7.4 U11.6 U11.7 U11.8 U11.13 U11.14 U12.4 U12.5 U12.6 U12.7 U15.3 U16.2 U16.3 U17.2 U17.3 U18.2 U18.3 U19.2 U19.3 U20.4 U21.4 U22.4 U23.4 |
| 3 | NET3 | 4 | C7.1 C8.2 C9.2 C10.2 C11.2 R5.1 R6.1 R7.1 R8.1 R9.1 R10.1 R11.1 R12.1 U2.1 U2.2 U2.10 U3.16 U3.23 U3.25 U4.1 U4.3 U4.10 U4.12 U4.17 U4.18 U8.1 |
| 73 | NET73 | 9 | C7.2 C8.1 C9.1 Q1.2 R2.2 R13.2 R24.2 R25.2 R28.2 R56.2 R57.1 R58.2 SW1.1 SW1.3 SW1.5 SW1.7 SW1.9 SW1.11 SW1.13 SW1.15 U2.20 U3.32 U4.19 U6.6 U7.8 U8.4 U8.8 |
| 74 | NET74 | 2 | U17.5 U22.3 U22.6 |
| 75 | NET75 | 6 | L7.1 U17.8 U18.8 U19.8 |
| 77 | NET77 | 2 | U18.5 U21.3 U21.6 |
| 78 | NET78 | 2 | U19.5 U20.3 U20.6 |
| 80 | NET80 | 2 | U16.5 U23.3 U23.6 |
| 93 | NET93 | 4 | C18.1 R25.1 U7.3 |
| 94 | NET94 | 4 | U2.3 U4.2 U8.3 |
| 95 | NET95 | 5 | C1.1 C19.1 C38.1 C39.1 C40.1 C41.1 L5.1 U16.6 U17.6 U18.6 U19.6 U20.8 U21.8 U22.8 U23.8 |
| 96 | NET96 | 4 | R52.1 R53.1 R60.2 U20.2 U21.2 U22.2 U23.2 |
| 97 | NET97 | 4 | R55.1 R56.1 R59.2 U20.5 U21.5 U22.5 U23.5 |
| 103 | NET103 | 2 | C11.1 U8.5 |
| 104 | NET104 | 2 | U2.18 U3.11 |
| 105 | NET105 | 2 | U2.11 U3.4 |
| 106 | NET106 | 2 | U2.12 U3.5 |
| 107 | NET107 | 2 | U2.13 U3.6 |
| 108 | NET108 | 2 | U2.14 U3.7 |
| 109 | NET109 | 2 | U2.15 U3.8 |
| 110 | NET110 | 2 | U2.16 U3.9 |
| 111 | NET111 | 2 | R6.2 SW1.12 U3.24 |
| 113 | NET113 | 4 | C13.1 U2.9 U4.11 U5.6 U5.7 |
| 114 | NET114 | 6 | C2.1 C3.1 C4.1 L1.2 L2.2 L10.2 R1.1 U4.9 U4.20 U5.8 U6.8 |
| 115 | NET115 | 4 | C17.1 R24.1 U7.5 |
| 116 | NET116 | 2 | R5.2 SW1.10 U3.27 |
| 117 | NET117 | 2 | R12.2 SW1.8 U3.3 |
| 118 | NET118 | 2 | R8.2 SW1.16 U3.29 |
| 119 | NET119 | 2 | R7.2 SW1.14 U3.28 |
| 123 | NET123 | 1 | Q6.3 |
| 124 | NET124 | 1 | L10.1 |
| 128 | NET128 | 2 | U2.6 U6.3 |
| 129 | NET129 | 2 | C5.2 R26.2 R27.1 U6.2 U7.2 U7.6 |
| 130 | NET130 | 3 | Q3.1 Q7.1 U6.1 |
| 131 | NET131 | 2 | Q5.1 Q9.1 U7.1 |
| 132 | NET132 | 2 | R18.2 U4.8 |
| 133 | NET133 | 2 | Q2.2 R23.1 |
| 134 | NET134 | 4 | C34.2 C35.2 L5.2 R44.2 R45.2 R46.2 U16.8 |
| 135 | NET135 | 1 | L7.2 |
| 136 | NET136 | 1 | U18.1 |
| 137 | NET137 | 1 | U19.1 |
| 139 | NET139 | 1 | U17.1 |
| 140 | NET140 | 1 | C28.2 C29.2 Q13.2 |
| 141 | NET141 | 1 | Q16.3 |
| 143 | NET143 | 1 | Q6.1 Q7.3 Q8.3 Q9.3 Q18.3 |
| 144 | NET144 | 1 | Q3.3 |
| 145 | NET145 | 1 | Q4.3 Q5.3 |
| 7 | NET7 | 0 | C30.1 R33.1 R34.2 U12.3 |
| 8 | NET8 | 0 | R32.1 R33.2 |
| 9 | NET9 | 0 | C6.1 CR4.1 R1.2 U6.5 |
| 10 | NET10 | 0 | C10.1 R3.1 U8.2 U8.6 |
| 11 | NET11 | 0 | L3.1 R38.1 R40.1 R41.2 |
| 12 | NET12 | 0 | C14.1 R15.1 R17.1 U5.5 |
| 13 | NET13 | 0 | C15.1 R18.1 U5.3 |
| 14 | NET14 | 0 | C16.1 Q2.1 R61.2 U5.1 |
| 15 | NET15 | 0 | C16.2 R19.1 R20.2 U5.2 |
| 16 | NET16 | 0 | C20.2 Q10.2 R32.2 R36.1 R38.2 U11.11 U12.8 |
| 17 | NET17 | 0 | C21.1 U15.8 |
| 18 | NET18 | 0 | C22.1 U15.4 |
| 19 | NET19 | 0 | C22.2 U15.2 |
| 20 | NET20 | 0 | C23.1 C24.1 L4.2 L6.2 Q12.1 Q12.2 R50.2 U15.5 |
| 21 | NET21 | 0 | C25.1 C26.1 C27.1 Q13.1 R50.1 R51.1 |
| 22 | NET22 | 0 | C31.1 L8.1 R35.2 U11.4 |
| 23 | NET23 | 0 | C32.1 R43.1 U11.10 |
| 24 | NET24 | 0 | C32.2 R40.2 R41.1 R42.2 |
| 25 | NET25 | 0 | C34.1 R39.1 R44.1 U11.5 |
| 26 | NET26 | 0 | C36.1 R45.1 R46.1 U16.1 |
| 27 | NET27 | 0 | C37.1 L9.1 |
| 28 | NET28 | 0 | C42.1 U16.4 |
| 29 | NET29 | 0 | C43.1 U17.4 |
| 30 | NET30 | 0 | C44.1 U18.4 |
| 31 | NET31 | 0 | C45.1 U19.4 |
| 32 | NET32 | 0 | CR5.1 L6.1 |
| 34 | NET34 | 0 | CR5.3 CR6.1 |
| 36 | NET36 | 0 | CR6.3 Q14.1 R47.2 |
| 37 | NET37 | 0 | L1.1 Q1.3 |
| 38 | NET38 | 0 | L2.1 Q2.3 R61.1 |
| 40 | NET40 | 0 | L4.1 Q13.3 |
| 41 | NET41 | 0 | L8.2 Q10.3 |
| 42 | NET42 | 0 | L9.2 Q16.2 R47.1 R48.1 R49.1 |
| 43 | NET43 | 0 | Q1.1 U6.7 |
| 44 | NET44 | 0 | Q4.1 Q8.1 U7.7 |
| 45 | NET45 | 0 | Q10.1 R36.2 R37.1 |
| 46 | NET46 | 0 | Q14.3 Q15.1 R48.2 |
| 47 | NET47 | 0 | Q15.3 Q16.1 R49.2 |
| 48 | NET48 | 0 | Q17.1 R57.2 R59.1 U20.1 U20.7 U21.1 U21.7 U22.1 U22.7 U23.1 U23.7 |
| 49 | NET49 | 0 | Q17.3 Q18.1 R58.1 R60.1 |
| 50 | NET50 | 0 | R2.1 R3.2 U8.7 |
| 51 | NET51 | 0 | R9.2 SW1.2 U3.2 |
| 52 | NET52 | 0 | R10.2 SW1.4 U3.31 |
| 53 | NET53 | 0 | R11.2 SW1.6 U3.30 |
| 54 | NET54 | 0 | R13.1 R14.2 |
| 55 | NET55 | 0 | R14.1 R15.2 |
| 56 | NET56 | 0 | R16.1 R17.2 |
| 57 | NET57 | 0 | R20.1 R21.2 |
| 58 | NET58 | 0 | R21.1 R22.2 |
| 59 | NET59 | 0 | R22.1 R23.2 |
| 60 | NET60 | 0 | R27.2 R28.1 |
| 61 | NET61 | 0 | R29.1 R30.1 R35.1 U11.3 U12.2 |
| 62 | NET62 | 0 | R30.2 R31.2 U11.12 |
| 63 | NET63 | 0 | R37.2 U12.1 |
| 64 | NET64 | 0 | R42.1 R43.2 |
| 65 | NET65 | 0 | R53.2 R54.2 |
| 66 | NET66 | 0 | R54.1 R55.2 |
| 69 | NET69 | 0 | U2.17 U3.10 |
| 76 | NET76 | 0 | U3.13 U4.7 |
| 79 | NET79 | 0 | U3.14 U4.6 |
| 81 | NET81 | 0 | U3.15 U4.5 |
| 83 | NET83 | 0 | U3.18 U4.4 |
| 84 | NET84 | 0 | U3.19 U4.16 |
| 85 | NET85 | 0 | U3.20 U4.15 |
| 86 | NET86 | 0 | U3.21 U4.14 |
| 87 | NET87 | 0 | U3.22 U4.13 |
| 33 | NET33 | 0 | CR5.2 |
| 35 | NET35 | 0 | CR6.2 |
| 39 | NET39 | 0 | L3.2 |
| 67 | NET67 | 0 | U2.4 |
| 68 | NET68 | 0 | U2.5 |
| 70 | NET70 | 0 | U2.19 |
| 71 | NET71 | 0 | U3.1 |
| 72 | NET72 | 0 | U3.12 |
| 82 | NET82 | 0 | U3.17 |
| 88 | NET88 | 0 | U3.26 |
| 89 | NET89 | 0 | U11.1 |
| 90 | NET90 | 0 | U11.2 |
| 91 | NET91 | 0 | U11.9 |
| 92 | NET92 | 0 | U15.1 |
| 98 | NET98 | 0 | U15.6 |
| 99 | NET99 | 0 | U15.7 |
| 100 | NET100 | 0 | U16.7 |
| 101 | NET101 | 0 | U17.7 |
| 102 | NET102 | 0 | U18.7 |
| 112 | NET112 | 0 | U19.7 |
| NC | 0 | none |  |

# Parts

## Part Summary and Pin Coverage

| Total number of parts | 152 |  |
| --- | --- | --- |
| Total number of part pins | 500 |  |
| Total number of inactive, no connect, SMD part pins | 28 | 6% |
| Total number active part pins | 472 | 94% |
| Number of accessible active part pins | 281 | 56% |
| Number of parts where 100% of the pins are accessible from bottom side | 38 | 25% |
| Number of parts where 90-99% of the pins are accessible from bottom side | 0 | 0% |
| Number of parts where 50-89% of the pins are accessible from bottom side | 69 | 45% |
| Number of parts where less than 50% of the pins are accessible from bottom side | 45 | 30% |
| Parts with 40 or more pins | 0 | 0% |
| Parts with 16-39 pins | 4 | 3% |
| Parts with 4-15 pins | 15 | 10% |
| 3-Pin Parts | 20 | 13% |
| 2-Pin Parts | 113 | 74% |
| 1-Pin Parts | 0 | 0% |

## Parts with 40 or more pins

No Parts for this part class.

## Parts with 16-39 pins

Pins inaccessible from bottom side shown in (bold).

An asterix "*" indicates an isolated pin.

A lower case "p" indicates potential power or ground pins.

| Reference Designator | Side | Total | Accessible Pins | % | Pin List |
| --- | --- | --- | --- | --- | --- |
| SW1 | T | 16 | 13 | 81% | 1 - (2) - 3 - (4) - 5 - (6) - 7 - 8 - 9 - 10 - 11 - 12 - 13 - 14 - 15 - 16 |
| U2 | T | 20 | 16 | 80% | 1p - 2p - 3 - (4*) - (5*) - 6 - 7p - 8p - 9 - 10p - 11 - 12 - 13 - 14 - 15 - 16 - (17) - 18 - (19*) - 20 |
| U3 | T | 32 | 16 | 50% | (1*) - (2) - 3 - 4 - 5 - 6 - 7 - 8 - 9 - (10) - 11 - (12*) - (13) - (14) - (15) - 16p - (17*) - (18) - (19) - (20) - (21) - (22) - 23p - 24 - 25p - (26*) - 27 - 28 - 29 - (30) - (31) - 32 |
| U4 | T | 20 | 12 | 60% | 1p - 2 - 3p - (4) - (5) - (6) - (7) - 8 - 9 - 10p - 11 - 12p - (13) - (14) - (15) - (16) - 17p - 18p - 19 - 20 |

## Parts with 4-15 pins

Pins inaccessible from bottom side shown in (bold).

An asterix "*" indicates an isolated pin.

A lower case "p" indicates potential power or ground pins.

| Reference Designator | Side | Total | Accessible Pins | % | Pin List |
| --- | --- | --- | --- | --- | --- |
| U5 | T | 8 | 4 | 50% | (1) - (2) - (3) - 4p - (5) - 6 - 7 - 8 |
| U6 | T | 8 | 6 | 75% | 1 - 2 - 3 - 4p - (5) - 6 - (7) - 8 |
| U7 | T | 8 | 7 | 88% | 1 - 2 - 3 - 4p - 5 - 6 - (7) - 8 |
| U8 | T | 8 | 5 | 62% | 1p - (2) - 3 - 4 - 5 - (6) - (7) - 8 |
| U11 | T | 14 | 5 | 36% | (1*) - (2*) - (3) - (4) - (5) - 6p - 7p - 8p - (9*) - (10) - (11) - (12) - 13p - 14p |
| U12 | T | 8 | 4 | 50% | (1) - (2) - (3) - 4p - 5p - 6p - 7p - (8) |
| U15 | T | 8 | 1 | 12% | (1*) - (2) - 3p - (4) - (5) - (6*) - (7*) - (8) |
| U16 | T | 8 | 5 | 62% | (1) - 2p - 3p - (4) - 5 - 6 - (7*) - 8 |
| U17 | T | 8 | 6 | 75% | 1* - 2p - 3p - (4) - 5 - 6 - (7*) - 8 |
| U18 | T | 8 | 6 | 75% | 1* - 2p - 3p - (4) - 5 - 6 - (7*) - 8 |
| U19 | T | 8 | 6 | 75% | 1* - 2p - 3p - (4) - 5 - 6 - (7*) - 8 |
| U20 | T | 8 | 6 | 75% | (1) - 2 - 3 - 4p - 5 - 6 - (7) - 8 |
| U21 | T | 8 | 6 | 75% | (1) - 2 - 3 - 4p - 5 - 6 - (7) - 8 |
| U22 | T | 8 | 6 | 75% | (1) - 2 - 3 - 4p - 5 - 6 - (7) - 8 |
| U23 | T | 8 | 6 | 75% | (1) - 2 - 3 - 4p - 5 - 6 - (7) - 8 |

## 3-Pin Parts

Pins inaccessible from bottom side shown in (bold).

An asterix "*" indicates an isolated pin.

A lower case "p" indicates potential power or ground pins.

| Reference Designator | Side | Pin List |  | Reference Designator | Side | Pin List |  | Reference Designator | Side | Pin List |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR4 | T | (1) - 2p - 3p |  | CR5 | T | (1) - (2*) - (3) |  | CR6 | T | (1) - (2*) - (3) |
| Q1 | T | (1) - 2 - (3) |  | Q2 | T | (1) - 2 - (3) |  | Q3 | T | 1 - 2p - 3* |
| Q4 | T | (1) - 2p - 3 |  | Q5 | T | 1 - 2p - 3 |  | Q6 | T | 1 - 2p - 3* |
| Q7 | T | 1 - 2p - 3 |  | Q8 | T | (1) - 2p - 3 |  | Q9 | T | 1 - 2p - 3 |
| Q10 | T | (1) - (2) - (3) |  | Q12 | T | (1) - (2) - 3p |  | Q13 | T | (1) - 2 - (3) |
| Q14 | T | (1) - 2p - (3) |  | Q15 | T | (1) - 2p - (3) |  | Q16 | T | (1) - (2) - 3* |
| Q17 | T | (1) - 2p - (3) |  | Q18 | T | (1) - 2p - 3 |  |  |  |  |

## 2-Pin Parts

Pins inaccessible from bottom side shown in (bold).

An asterix "*" indicates an isolated pin.

A lower case "p" indicates potential power or ground pins.

| Reference Designator | Side | Pin List |  | Reference Designator | Side | Pin List |  | Reference Designator | Side | Pin List |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C1 | T | 1 - 2p |  | C2 | T | 1 - 2p |  | C3 | T | 1 - 2p |
| C4 | T | 1 - 2p |  | C5 | T | 1p - 2 |  | C6 | T | (1) - 2p |
| C7 | T | 1p - 2 |  | C8 | T | 1 - 2p |  | C9 | T | 1 - 2p |
| C10 | T | (1) - 2p |  | C11 | T | 1 - 2p |  | C13 | T | 1 - 2p |
| C14 | T | (1) - 2p |  | C15 | T | (1) - 2p |  | C16 | T | (1) - (2) |
| C17 | T | 1 - 2p |  | C18 | T | 1 - 2p |  | C19 | T | 1 - 2p |
| C20 | T | 1p - (2) |  | C21 | T | (1) - 2p |  | C22 | T | (1) - (2) |
| C23 | T | (1) - 2p |  | C24 | T | (1) - 2p |  | C25 | T | (1) - 2p |
| C26 | T | (1) - 2p |  | C27 | T | (1) - 2p |  | C28 | T | 1p - 2 |
| C29 | T | 1p - 2 |  | C30 | T | (1) - 2p |  | C31 | T | (1) - 2p |
| C32 | T | (1) - (2) |  | C34 | T | (1) - 2 |  | C35 | T | 1p - 2 |
| C36 | T | (1) - 2p |  | C37 | T | (1) - 2p |  | C38 | T | 1 - 2p |
| C39 | T | 1 - 2p |  | C40 | T | 1 - 2p |  | C41 | T | 1 - 2p |
| C42 | T | (1) - 2p |  | C43 | T | (1) - 2p |  | C44 | T | (1) - 2p |
| C45 | T | (1) - 2p |  | L1 | T | (1) - 2 |  | L2 | T | (1) - 2 |
| L3 | T | (1) - (2*) |  | L4 | T | (1) - (2) |  | L5 | T | 1 - 2 |
| L6 | T | (1) - (2) |  | L7 | T | 1 - 2* |  | L8 | T | (1) - (2) |
| L9 | T | (1) - (2) |  | L10 | T | 1* - 2 |  | R1 | T | 1 - (2) |
| R2 | T | (1) - 2 |  | R3 | T | (1) - (2) |  | R5 | T | 1p - 2 |
| R6 | T | 1p - 2 |  | R7 | T | 1p - 2 |  | R8 | T | 1p - 2 |
| R9 | T | 1p - (2) |  | R10 | T | 1p - (2) |  | R11 | T | 1p - (2) |
| R12 | T | 1p - 2 |  | R13 | T | (1) - 2 |  | R14 | T | (1) - (2) |
| R15 | T | (1) - (2) |  | R16 | T | (1) - 2p |  | R17 | T | (1) - (2) |
| R18 | T | (1) - 2 |  | R19 | T | (1) - 2p |  | R20 | T | (1) - (2) |
| R21 | T | (1) - (2) |  | R22 | T | (1) - (2) |  | R23 | T | 1 - (2) |
| R24 | T | 1 - 2 |  | R25 | T | 1 - 2 |  | R26 | T | 1p - 2 |
| R27 | T | 1 - (2) |  | R28 | T | (1) - 2 |  | R29 | T | (1) - 2p |
| R30 | T | (1) - (2) |  | R31 | T | 1p - (2) |  | R32 | T | (1) - (2) |
| R33 | T | (1) - (2) |  | R34 | T | 1p - (2) |  | R35 | T | (1) - (2) |
| R36 | T | (1) - (2) |  | R37 | T | (1) - (2) |  | R38 | T | (1) - (2) |
| R39 | T | (1) - 2p |  | R40 | T | (1) - (2) |  | R41 | T | (1) - (2) |
| R42 | T | (1) - (2) |  | R43 | T | (1) - (2) |  | R44 | T | (1) - 2 |
| R45 | T | (1) - 2 |  | R46 | T | (1) - 2 |  | R47 | T | (1) - (2) |
| R48 | T | (1) - (2) |  | R49 | T | (1) - (2) |  | R50 | T | (1) - (2) |
| R51 | T | (1) - 2p |  | R52 | T | 1 - 2p |  | R53 | T | 1 - (2) |
| R54 | T | (1) - (2) |  | R55 | T | 1 - (2) |  | R56 | T | 1 - 2 |
| R57 | T | 1 - (2) |  | R58 | T | (1) - 2 |  | R59 | T | (1) - 2 |
| R60 | T | (1) - 2 |  | R61 | T | (1) - (2) |  |  |  |  |

## 1-Pin Parts

No Parts for this part class.